MATLAB m.file for the EMG signal processing

% This routine was written for the educational purposes.

% Basic EMG Signal Processing Techniques

%% EMG and Force relationship for wrist extensor muscle

% data name = nesibe1.txt; sampling frequency = 100 Hz.

% first column: time

% second column: force (N)

% third column: EMG (mV)

clear;clc;close all

load('nesibe1.txt')

time = nesibe1(:,1);

Force = nesibe1(:,2);

EMG.raw = nesibe1(:,3);

EMG.det = detrend(EMG.raw); % Elimination of the DC component of the signal

figure

subplot(2,1,1); plot(time,EMG.raw)

xlabel('time (s)'); ylabel('EMG (mV)'); legend('Raw EMG')

subplot(2,1,2); plot(time,EMG.det)

xlabel('time (s)'); ylabel('EMG (mV)'); legend('detrended EMG')

figure

subplot(2,1,1);plot(time,Force)

ylabel('Force (N)'); xlabel('time (s)')

subplot(2,1,2);plot(time,EMG.det)

ylabel('EMG (mV)'); xlabel('Time (s)')

%% Construction time series if needed

fs = 100; % sampling frequency (Hz)

fn=fs/2; % Nyquist Frequency (Hz) - (1/2 of Sampling Frequency)

step_data = 1/fs; % Step amount

t = 0:step_data:(length(EMG.raw)-step_data)/fs;

%% Rectification

% Half-wave rectification

for i = 1:length(EMG.det);

if EMG.det(i) > 0;

EMG.half(i)= EMG.det(i);

else

EMG.half(i)= 0;

end

end

%%%%% Alternative method I for half-wave rectification %%%%%

%     Make the negative values using function "find"

%     ind = find(EMG.det ~= abs(EMG.det)); % Mutlak değeri kendisine eşit olmayan verilerin indisini bulalım.

%     EMG.det(ind) = 0; % Mutlak değeri kendisine eşit olmayan verileri 0'a eşitleyelim.

%     plot(EMG.det)

%%%%% Alternative method II for half-wave rectification %%%%%

% Logical indexing

% ind1 = EMG < 0;

% EMG(ind1) = 0;

% plot(EMG)

% Full wave rectification

EMG.full = abs(EMG.det);

figure

subplot(4,1,1)

plot(time,Force); ylabel('Force (N)'); xlabel('time (s)')

subplot(4,1,2)

plot(time,EMG.det); xlabel('time (s)'); ylabel('EMG (mV)');legend('Raw EMG')

subplot(4,1,3)

plot(time,EMG.half);xlabel('time (s)'); ylabel('EMG (mV)');legend('Half-wave rectification')

subplot(4,1,4)

plot(time,EMG.full);xlabel('time (s)'); ylabel('EMG (mV)');legend('Full-wave rectification')

%% Filtering an EMG signal

% Let's filter the EMG signal with band-pass filter (six

% order Butterworth with a cut-off of 20-50 Hz).

low_freq = 20;

high_freq = 40;

[b,a] = butter(6,([low_freq high_freq]/fn));% Coefficients for filter

EMG.filtered = filtfilt(b,a,EMG.det); %Filter data forward and backward to

% eliminate phase shift

figure

plot(time,EMG.det);hold on;

plot(time,EMG.filtered,'r');legend('Unfiltered EMG','Filtered EMG')

%% Linear Envolope of EMG

% Requires two-step process: full-wave rectification + low-pass filter

% Constructing a low pass filter. We will evaluate the effects of two

% frequency value ın the signal smoothing.

fc = [2 10]; % Cut off frequencies (Hz)

[b,a]=butter(2,fc(1)/fn,'low');

% Filtering the signal

EMG.filtered.FourHz = filtfilt(b,a,EMG.full);

[b,a]=butter(2,fc(2)/fn,'low');

% Filtering the signal

EMG.filtered.TenHz = filtfilt(b,a,EMG.full);

figure

subplot(4,1,1)

plot(time,Force); ylabel('Force (N)'); xlabel('time (s)')

subplot(4,1,2)

plot(time,EMG.full); xlabel('time (s)'); ylabel('EMG (mV)');legend('Raw EMG')

subplot(4,1,3)

plot(time,EMG.filtered.TenHz,'k','LineWidth',2);

legend('Linear envelope - 10Hz');

subplot(4,1,4)

plot(time,EMG.filtered.FourHz,'r','LineWidth',2);legend('Linear envelope - 4Hz')

figure;

subplot(2,1,1);

plot(time,Force/max(Force));

hold on

plot(time,EMG.filtered.TenHz/max(EMG.filtered.TenHz),'k','LineWidth',2);

title('Direct comparison of force and EMG', 'Fontsize',14)

xlabel('time (s)'); legend('Force', '10Hz linear enveloped EMG')

subplot(2,1,2)

plot(time,Force/max(Force));

hold on

plot(time,EMG.filtered.FourHz/max(EMG.filtered.FourHz),'r','LineWidth',2);

xlabel('time (s)');legend('Force', '2Hz linear enveloped EMG')

%% Average rectified value (ARV) or Mean amplitude value (MAV)

% ARV is the mean value of the rectified EMG over a time interval T

T = 0.25; % Width of time segments(s)

n_data = T*fs; %number of segmented data

e = 0;

for i = 1:length(EMG.full)/n_data;

for j = (i-1)*n_data+1:i*n_data+1;

e(j+1) = e(j) + (EMG.full(j));

end

EMG.ARV(i) = e(j+1)/n_data;

e(j) = 0;

end

i = 1:length(EMG.full)/length(EMG.ARV):length(EMG.full);

figure

subplot(2,1,1)

plot(time,EMG.full);hold on

plot(i/fs,EMG.ARV,'k','Linewidth',2);legend('Rectified EMG','Average rectified value of EMG')

xlabel('time (s)')

subplot(2,1,2)

plot(time,Force/max(Force));hold on

plot(i/fs,EMG.ARV/max(EMG.ARV),'k','Linewidth',2);legend('Force','Average rectified value of EMG')

title('Direct comparison of force and EMG ARV', 'Fontsize',14); xlabel('time (s)')

%% RMS of a EMG signal

T = 0.25; % Width of time segments(s)

n_data = T*fs; %number of segmented data

e = 0;

for i = 1:length(EMG.full)/n_data;

for j = (i-1)*n_data+1:i*n_data+1;

e(j+1) = e(j) + (EMG.full(j))^2;

end

EMG.RMS(i) = sqrt(e(j+1)/n_data);

e(j) = 0;

end

i = 1:length(EMG.full)/length(EMG.RMS):length(EMG.full);

figure

subplot(2,1,1)

plot(time,EMG.full);hold on

plot(i/fs,EMG.RMS,'k','Linewidth',2);legend('Rectified EMG','EMG RMS');xlabel('time (s)')

subplot(2,1,2)

plot(time,Force/max(Force));hold on

plot(i/fs,EMG.RMS/max(EMG.RMS),'k','Linewidth',2);

legend('Force','EMG RMS');xlabel('time (s)')

title('Direct comparison of force and EMG RMS', 'Fontsize',14); xlabel('time (s)')

%% Integrated EMG

%% Power spectra of raw EMG

% data name = data_EMG; sampling frequency = 1000 Hz.

EMG1.raw = load('data_emg.txt'); % Raw EMG

EMG1.det = detrend(EMG1.raw);

fs1 = 1000; % sampling frequency (Hz)

time1 = 8.08; % recording duration (s)

N1 = fs1*time1 ; % number of data

t1 = 0:1/fs1:time1-1/fs1; % time scale

EMG1.fft = fft(EMG1.det);

EMG1.abs = abs(EMG1.fft);

EMG1.pds = EMG1.abs.^2/N1; %periodogram

f1=0:fs1/N1:fs1/2;

figure

plot(f1(1:N1/2),EMG1.pds(1:N1/2))

xlabel('Frequency (Hz)'); ylabel('Power mV^{2}/Hz');legend('Power Spectrum of EMG');